# Quick Hangar

- Menu: File Geo-location Add location

- Type airport ICAO

- Find hangar

- Zoom in all the way

- Click Select region

- Click Grab, wait a few seconds

- You now have an aerial image in SketchUp

- Click on Top view (Views toolbar)

- Right-click  Unlock

- Move tool to one of the FRONT corners of the hangar on Origin (crossing of colored lines)

- Rotate tool

  - Click on Origin

  - Click on other FRONT corner with guide right on the edge

  - Move onto red axis and click

- Rectangle tool, draw the edges starting from Origin

- Tape tool, draw guides to find middle of building (or front). You see blue point when in middle of an edge.

- Press Control + A, selects all including the map

- Move tool from center to Origin point, including the map.

- I prefer my models to be centered on the center of the building but if you prefer the front you can do that too. Since you are placing your models in ADE I think it would be more convenient for you.

- Front view

- Hide map, Edit  Hide (I have this one mapped to H key, Ctrl+H to unhide all)

- Tape tool, draw vertical guide on front edge and center

- Draw horizontal guides on front

- Pull tool, up to 7m

- Line tool for the roof

- Pull tool to pull the roof from behind

That’s it for the basics, quick and precise since we used Geo-location.

The problem is always to figure out the height. For standard hangars I go with 7m/10m. Habitation, commercial buildings, etc. bottom and top floor 4m, other floors 3m. Standard door 0.90 x 2.10m.

Now let’s put some fancy doors on the hangar.

- Remove all guides (I got this one mapped to W  key)

- Tape tool, draw a vertical guide in front center, an horizontal one at height 5.6m

- Plan how many door panels you are going to use. Here I’ll have 6 panels of 3m each.

- Tape tool from center draw left and right vertical guide at 9m

- Rectangle tool, draw out-shape of the doors

- Pull tool, push the whole door in by .30m

- Delete all guides

- From edges of the door draw guides going inward at 3m

- Rectangle tool, draw the outer 3m panels

- Pull tool, push center doors inward by 0.30m again

- Delete all guides and repeat the 3m process for the inner doors

- Line tool to draw the center

- Don’t forget to delete the base inside

- Also Edit  Unhide  All

- Delete the Geo-location map

- Bucket tool, remove the map texture from “In Model” textures

Now let’s get even fancier and learn to use the Follow Me gadget. Say you want to have a pipe running on the side of the building, not an easy part to draw manually. But with the Follow Me tool it’s a brise.

- Tape tool, create horizontal and vertical guides where you want the center of the pipe to be.

- Rectangle tool, draw the rectangle

- Tape tool, create a couple markers equidistant from the outside corner

- 2 Point Arc tool, draw an arc on the rectangle. Click on top edge, move to right edge, ideally the arc will turn purple, click on right edge, click again to finalize.

- Delete all guides, delete bottom and left edges and top right corner

- From contact point on wall, use Circle tool to draw a circle. Mine is 0.25m in radius

- Select the whole line by keeping Control key down

- Select the Follow Me tool

- Click on the surface inside the circle and you get:

- Revert faces on the pipe

Let’s pretend we have a template and that vaguely what it would look like. Not bad for a very quick model design.